%Bode Plots and RC Circuit rc_w1000cutoff

%  bode1and2tlh.m Plot of 1st Order System

% Obtain the Bode plot of the system given by the transfer function

%           H(s) =1/(2s + 1)

%  Convert the transfer function by substituting s = iw

%           H( jw) =  1/(2jw + 1)  if doing this by hand.

%   Note H(0)= 1 (0dB). H(1/2)= -3dB. H(10*(1/2)=-20 dB

%    wc= 1/2 is the break point or cutoff radian frequency.

num = 1;          % 1/(2s+1) as Laplace Transfer Function

den = [2 1];

sys = tf(num,den);

figure(1)

bode(sys),grid

title('Bode Plot for 1st order system, \omega_c =0.5r/s')

%

% Obtain the bode plot of the system given by the transfer function

% hH(s) =   4 /(s^2 + s + 4)  This is 4/(s^2+ 2*zeta*wn + wn^2)

num = 4;

den = [1 1 4];      % wn =2 2*zeta*wn=1 so that zeta=1/4

sys = tf(num,den);

figure(2)

bode(sys), grid

title('Bode Plot for 2st order system, \omega_n =2 r/s \zeta =.25')

%   Note H2(0)= 1 (0dB).  H2(10*(2)~=-40 dB

% rc_w1000cutoff  See Example 8.8 Harman Eq. 8.48

clear,clc,clf

RC = 0.001;             % w_c=1/RC = 1000 radians/sec

w = 0:50:5000;          % Plot range in omega

H = (1/RC)./(j*w+1/RC);

magH = abs(H); angH = 180*angle(H)/pi;

figure(1),subplot(211),plot(w,magH)

title('Series RC, \omega_c = 1000 r/s')

xlabel('Frequency (rad/sec)')

ylabel('|H|'),grid

% the following adds lines that identify the points

%   where |H|=.707

hold on

plot([800 1000],[.707 .707],'r:')   %,...

hold off

text(200,.707,'0.707'),subplot(212),plot(w,angH);

axis([0 5000 -90 0])

xlabel('Frequency (rad/sec)'),ylabel('Angle(H), degrees'),grid

% Bode Plot   H= (1/RC)/(s+1/RC)  1/RC=1000

figure(2)

num=1000

den=[1 1000]

sysrc=tf(num,den)

bode(sysrc),grid

title('Bode diagram for RC circuit \omega_c = 1000 r/s')

hold

gtext('Use Tools/Data Cursor for Values')

hold off