% smootherEx10_7filt.m A smoothing filter defined as

%   y(n) = a*y(n-1) +(1-a)*x(n) , y(-1)=0

%   x(n) is input signal, y(n) is smoothed output

%

% Test signal is sin(w*t) with random noise

%  INPUT:  Weighing factor a

%  OUTPUT: Plot of x and y

%

clear, clf

w=2*pi/5;

t = linspace(0,10,100);     % Time steps

s = sin(w*t);               % Noiseless signal

% Add random noise

len=size(t);

na = 0.1;                   % Noise amplitude

noise = na*(rand(len)-.5);   % (-.05 to +.05)

x = s + noise;

%

% Weighing factor

a = input('Weighing factor a= ')

%

y(1)=(1-a)*x(1);

for I=2:100

y(I) = a*y(I-1) + (1-a)*x(I);  % Digital Filter

end

figure(1),plot(t,x,t,y),grid

xlabel('Time'), ylabel('Signals')

title(['Effect of Smoothing Filter Ex 10.7, a = ', num2str(a)])

legend('Input x','Output y')

% Filter

avec=[1 -a]

bvec=[1-a]

yfilt=filter(bvec,avec,x)

figure(2),plot(t,x,t,yfilt),grid,xlabel('Time'),axis([0 10 -1.5 1.5])

title(['Effect of MATLAB Filter Ex 10.7, a = ', num2str(a)])

legend('Input x','Output y')